# Table

# List of Allocated Frequency Bands, Transmission power and Transmission Methods for Amateur Radio Stations.

| Frequency band distributed (MHz) | Transmission power*Note 1 |  |  | Transmission methods *Note 2 |
| --- | --- | --- | --- | --- |
|  | Class 1 | Class 2 | Class 3 |  |
| 0.1357-0.1378 | < 1W | < 1W |  | Telegraph, radioteletype, data |
| 1.8-1.9 | < 1500W | < 600W |  | Telegraph, telephone, video, radioteletype, data |
| 3.5-3.5125 | < 1500W | < 200W |  | Telegraph, radioteletype, data |
| 3.55-3.5625 | < 1500W | < 200W |  | Telegraph, telephone, video, radioteletype, data |
| 7.0-7.025 | < 1500W | < 600W |  | Telegraph, radioteletype, data |
| 7.025-7.2 | < 1500W | < 600W |  | Telegraph, telephone, video, radioteletype, data |
| 10.13-10.15 | < 200W | < 200W |  | Telegraph, radioteletype, data |
| 14.0-14.35 | < 1500W | < 600W |  | Telegraph, telephone, video, radioteletype, data |
| 18.068-18.168 | < 1500W | < 600W |  | Telegraph, telephone, video, radioteletype, data |
| 21.0-21.025 | < 1500W | < 600W |  | Telegraph, radioteletype, data |
| 21.025-21.2 | < 1500W | < 600W |  | Telegraph, radioteletype, data |
| 21.2-21.45 | < 1500W | < 600W |  | Telegraph, telephone, video, radioteletype, data |
| 24.89-24.99 | < 1500W | < 600W |  | Telegraph, telephone, video, radioteletype, data |
| 28.0-29.7 | < 1500W | < 600W |  | Telegraph, telephone, video, radioteletype, data, beacon, FM telephone, relay, satellite amateur |
| 50.0-50.15 | < 1500W | < 600W | < 25W | Telegraph, modulated telegraph, telephone, video, radioteletype, data, satellite amateur |
| 144-146 | < 200W | < 200W | < 25W | Telegraph, modulated telegraph, telephone (SSB), video, radioteletype, data, (EME), testing, satellite amateur |
| 430-432 | < 200W | < 200W | < 25W | Telegraph, modulated telegraph, telephone (SSB), video, radioteletype, data, spread spectrum, EME, testing |
| 432-440 | < 200W | < 200W | < 25W | modulated telegraph, telephone, video, radioteletype, data, spread spectrum, (EME), telephone(NFM), emergency, distress signal, relay, satellite amateur, testing |
| 1,260-1,265 | < 200W | < 200W |  | Telegraph, modulated telegraph, telephone, video, radioteletype, data, spread spectrum, testing, satellite amateur |
| 2440-2450 | < 200W | < 200W |  | Telegraph, modulated telegraph, telephone, video, radioteletype, data, spread spectrum, testing, impulse, satellite amateur |
| 47,000-47,200 | < 200W | < 200W |  | Telegraph, modulated telegraph, telephone, video, radioteletype, data, spread spectrum, testing, impulse, satellite amateur |
| 248,000-250,000 | < 200W | < 200W |  | Telegraph, modulated telegraph, telephone, video, radioteletype, data, spread spectrum, testing, impulse, satellite amateur |

*Note 1: The transmission power of an amateur radio station refers to Equivalent Isotropic Radiated Power (EIRP.) for the frequency band between 0.1357 and 0.1378MHz and peak envelope power (PEP) for other frequency bands.

*Note 2: The frequency range for the transmission methods used in each of the distributed frequency band shall adhere to the IARU Region 3 Band Plans Recommendation of International Amateur Radio Union.